Multiscale Assessment of Industrial Atmospheric Emissions, Urban Air Quality Degradation, and Population-Level Health Exposure Using AERMOD Simulation: A Case Study of the South Pars Energy Zone

Ali Mehri

Chemical and Petroleum Engineering Department, Sharif University of Technology, Tehran, Iran

Reza Rabie

Graduate Faculty of Environment, University of Tehran, Tehran, Iran

## Abstract

Continuous atmospheric releases from large-scale industrial complexes constitute a persistent driver of air quality degradation and population exposure in adjacent urban environments. Accurate characterization of pollutant transport, dispersion behavior, and spatial concentration gradients is therefore essential for advanced environmental health assessment and evidence-based regulatory decision-making. This study investigates atmospheric dispersion and ground-level concentration fields of carbon monoxide, nitrogen dioxide, and sulfur dioxide within residential receptor zones surrounding the South Pars Energy Zone in southern Iran. Emission inventories were developed using detailed operational and geometric characteristics of ninety elevated stacks and twenty-six flaring sources, integrated within the AERMOD regulatory dispersion modeling framework under site-specific meteorological and surface conditions. Model-derived concentration fields were evaluated through comparison with ambient measurements collected at forty-five strategically distributed locations to ensure robustness of exposure representation. Results demonstrate pronounced spatiotemporal heterogeneity and pollutant-specific behavior across multiple averaging periods relevant to human exposure. Maximum concentrations reached 2428 and 418 ug/m3 for one-hour and eight-hour carbon monoxide exposure, respectively. Annual and short-term nitrogen dioxide levels attained peak values of 210 and 62 ug/m3, exceeding applicable ambient air quality thresholds. In contrast, sulfur dioxide concentrations remained within regulatory limits, supporting source–receptor and risk interpretation.

## Keywords: Industrial air emissions, AERMOD simulation, Urban air quality, Atmospheric dispersion modeling, Population health exposure

- Introduction

The industrial revolution caused world structures to undergo a profound transformation. A key consequence of the industrial revolution has been the increase in productivity and production of higher-quality goods, the reduction of prices and the increase in income levels, and the motivation for migration from rural to urban areas.[1], [2], [3]. In spite of this, these changes are much broader and have multiple facets that mutually affect each other. Pollution caused by industrial expansion in recent years has posed a severe threat to the health of many people [4], [5]. The phenomenon of air pollution is one of the results of industrial development, which increases day by day with the increase in population, expansion of urbanization, and the consumption of fossil fuels, and considering the risks that this pollution has for the health of people living in polluted areas [6], [7], [8]. In order to prevent production or reduce risk, we must be aware of this issue. There are around 3.1 million premature deaths caused by air pollution each year because of the severity of these risks [5], [9]. Every year, air pollution causes concerns and complications in many parts of the world. Consequently, air quality control strategies for decision-making bodies play a crucial role in the urban planning process [8], [10]. Therefore, the development and evaluation of air pollution control strategies have become a key component of urban planning due to the maturing attitude of city managers [11], [12], [13].

Refineries play a crucial role in the current oil and gas industry processes and are an important component of the global supply chain. Every day, these industrial sites emit a large amount of pollutants into the atmosphere [14], [15], [16]. Refineries produce most of these pollutants through combustion, cracking, reforming, isomerization, cooling, and recycling [17]. However, the occurrence of unwanted leaks, fire and explosion incidents, as well as process changes, may cause the release of high concentrations of pollutants in short periods [14], [18]. The main pollutants caused by these industries include carbon monoxide (CO), nitrogen oxides (NOx), sulfur oxides (SOx), ozone (O3), suspended particles (PM), and lead (Pb) [10], [19].

Understanding how pollutants disperse in the atmosphere is necessary to achieve legal goals and minimize adverse health effects caused by these pollutants [13], [20]. In the air, the gas is dispersed by the wind once it is released from the source. Thus, the emission point is the place where their maximum concentrations are observed. As a result of turbulent mixing with air, the concentration in the wind direction is relatively low. Dispersion of gases is affected by several parameters. Among them are wind speed and direction, atmospheric stability, ground conditions (buildings, mountains, trees, and water), and the height of the release of gases from the ground (chimney height). Due to topographical conditions and a lack of equipment, industries often face the difficulty of determining the amount of  pollutants at very far distances. This makes it difficult to determine the amount of pollutants present in remote areas. Therefore, simulation software is used in these situations to calculate the amount of pollutants in different areas and then compare them to measured results in inaccessible areas [21], [22].

The use of simulations in predicting the state of emissions of pollutants from industries is a developing field [23], [24]. In many cases, simulation saves time and money. As well, the simulation results provide higher levels of analysis [25]. In a study by Ali Mott et al [26], evaluated the dispersion of odorous pollutants in different areas around a refinery unit without direct measurements using a simulated model. This simulation was implemented using meteorological information about the region, pollution concentrations in 62 specific points, and topographical information about the region. AERMOD is also used to process data and generate output maps. In another study, Dusica J. Pesic and colleagues [27] examined how pollutants spread in a column of smoke caused by a fire in a city building. This research aims to evaluate the health and safety risks for people living in nearby buildings and passersby. Buildings were considered to be on fire in certain areas. A fire dynamics simulator (FDS) was used to model pollutant emissions in the smoke column and study the effects of wind flow on pollution transfer and spread. In another study, Amit Munjal and his colleagues [28] studied the comparison between the amount of suspended solid particles in highway toll stations and other parts of the highway. To carry out this study, samples were taken from 500-meter intervals of toll stations and the length of the highway. To predict the pollutant concentration level at other toll stations and high traffic points along the route, the research group used software simulation of solid pollutant emission using AERMOD software. According to the obtained results, the state of pollutants caused by cars passing along the highway was favorable.

The correctness and accuracy of the results obtained from various simulation packages is a challenge in this field [25]. Interpolation is another method used to determine the concentration and dispersion of different pollutants. In these methods, several samples from the study area are selected based on the appropriate distribution. In the next step, the concentration and dispersion of pollutants are predicted by various interpolation methods such as IDW, SPLINE, and KRIGING [29], [30], [31]. With these methods, continuous results are presented using discrete data. Awkash Kumar et al.[32] measured the concentration of three pollutants, SO2, NO2, and PM10 in 12 locations in Mumbai. Then, the results of sampling were entered into the ArcGIS software and interpolation of pollutant concentration and distribution was performed for two pollutants SO2 and NO2 using the IDW method and PM10 pollutant using the KRIGING method. As a final step, 11 interpolated points were sampled for validation of the method's results. According to the obtained results, sampling and interpolation results are in agreement. In another study, Li-HAN and colleagues [33]

used the interpolation method with the help of ArcGIS software to evaluate the reduction of air pollution in China as a result of the closure of industries during the Covid-19 pandemic, and by receiving information on pollutants and  using the KRIGING interpolation method determined the reduction of three pollutants PM2.5, PM10, O3 in the study area.

In this study, CO, NO2, and SO2 dispersion in Siraf city, a settlement in the vicinity of South Pars Energy Zone, the largest oil and gas facility in Iran, were studied with both field and simulation methods. To carry out this study, it was necessary to measure the amount and volume of pollution entering the air from 90 chimneys and 27 burners. These were found to be the main sources of pollutants entering the air from these sources. Based on the topography and climate data of the region, and using AERMOD software to assess the distribution of pollutants in different areas and to determine the amount of pollution dispersed, the state of concentration of pollutants was calculated and shown in detail. In the following, pollutants were measured in 19 points between Asaluyeh and Siraf refineries, as well as 29 points within Siraf's urban areas. By using the obtained data and the KRIGING method in ArcGIS software, the distribution and concentration maps of pollutants were generated. In many studies, the state of dispersion of pollutants has been measured only in certain areas of urban areas and based on the limited results obtained, a decision is made regarding the assessment of the environmental status of pollutants in the entire region. In many situations, a terrestrial effect can affect pollutant concentrations. In this study, first, the method of distribution and concentration of pollutants in all points of the study area is determined using simulation using AERMOD software, and then a map of the distribution of the pollutants in all investigated areas is drawn from the simulation. On the other hand, the results obtained from the simulation section are validated using the field method and sampling from the studied areas, and the accuracy of the simulation is determined. Therefore, the results of this study are such that they provide the possibility of any analysis of the state of pollutants in the urban environment and the evaluation of the consequences caused by them comprehensively and accurately, and also determine the accuracy and correctness of the simulation results.

In the following, first, a description of the situation of the South Pars energy region is presented and the pollutant sampling method in some parts of this region is explained. As the main part of the study, using topographical and meteorological data and using the AERMOD software package, the state of distribution of pollutants and their concentration in the entire study area is presented. Finally, while presenting and interpreting the results, a comparison is made between the results obtained from the field study and the simulation.

2. Materials and methods

2.1. area of ​​study

The city of Siraf is located in the southern part of Iran. Siraf's area is 1340 hectares, while the approved area is 8000 hectares. A map of Siraf city and its surrounding cities can be found in Figure 1.

Figure 1- The location of Study area on the map

A distance of about 10 kilometers separates Siraf city from the Persian Gulf. The point can be affected by land-to-land and sea-to-land winds during the day and at night. According to the figure, the Zagros Mountain range defines

the northern boundaries of Siraf city and the city's area. This may be one of the reasons why pollutants accumulate in this area under different atmospheric conditions. Furthermore, the existence of this mountain range can cause mountain winds to affect the plane and its air conditioning. Asaluyeh refineries, located near the boundary of Siraf city, are bounded by the Zagros Mountain range and the Persian Gulf from the north and south. It is the land of Mahouri Hill that separates the plains from the mountains. Approximately 45% of the direct study area is in the zero to 200 meter altitude class. It should be noted that the highest point inside the site is 30 meters. Towards the north of the region, the height has grown and reaches 1200 meters above sea level. Pollutant distribution and concentration are fundamentally affected by the conditions of the prevailing winds in the region. Wind conditions can be determined based on two variables: wind direction and wind speed. From the national meteorological station in Asaluyeh, the required data was gathered in order to evaluate and combine the wind situation. Data analysis was performed on a wind rose diagram based on the results of data processing. Other meteorological information about the region was received from the regional meteorological center from 2010 to 2020, including maximum temperatures, minimum temperatures, average temperatures, and relative humidity.

2.2. Sampling

During this descriptive-cross-sectional study, the location of sampling was at the refinery pollutant emissions, within the city of Siraf, and the distance between the refinery site and Siraf city. Simulations were performed with AERMOD software based on samples taken from refinery pollutants. In addition, based on data obtained from sampling within Siraf city, the distance between the refinery and Siraf city has been interpolated. A high-volume pump, cyclone filter holder, and fiberglass filter were used to collect air. MRU&SKC's Delta-2000CD device was employed in this study. Electrochemical cells were used to measure pollution in this field study. Air sampling continued for one month in August 2021. Sampling was done seasonally at a speed of 0.4 m/s. To prevent the influence of humidity on the evaluation process, filters were placed in a desiccator 24 hours before and after sampling and then weighed. Figure 2 shows the location of Siraf city limits and boundaries, as well as the measured points in the field, which are schematically shown. Also, the topography of the study area of Siraf city is depicted. A total of 162 pollutant emission points were measured, including 90 chimney points and 27 burner points in the Asaluyeh refinery area. Places outside the refinery area were also selected. There were 29 points within Siraf's study area that were more concentrated within the city of Siraf. The selection of these points was based on the concentration of population and density of building fabric in these areas. Another 16 sampling points were selected between the Asaluyeh refineries and the study area in the direction of the prevailing wind. By selecting these points, it has been possible to simulate the distribution of pollutants in both longitudinal and transverse directions accurately.

(a)                                                                                          (b)

Figure 2- Siraf area and sampling points a) Sampling location in Siraf city and the distance between the city and the refinery site b) Refinery pollutant measurement location

In order to compare the data distribution with the standard normal distribution, the Q-Q plot was used. The Q-Q plot in statistics is a probability chart, which is a graphical method for comparing two probability distributions by plotting two quantiles against each other. First, a set of intervals for the quantiles is selected. The point (x,y) on the graph corresponds to one of the quantiles of the second distribution (y-coordinate) plotted against the same quantile from the first distribution (x-coordinate). Q-Q plots are commonly used to compare sets of data. are used for a theoretical model. This graph can provide the possibility of evaluating the "fit" that is a graph instead of reducing it to a statistical summary. QQ plots are also used to compare two theoretical distributions. This statistical method is used to ensure that the accuracy of the sampled data was used.

2.3. Modeling the distribution of pollutants with AERMOD

The diffusion model used in this study is the AERMOD model. AERMOD software is one of the software approved by the US Environmental Protection Agency, which is in the category of preferred and recommended models of this organization [34]. Among the limitations of this model, we can mention very low wind conditions (calm) and extreme changes in weather conditions. In this model, it is assumed that the concentration distribution in the stable boundary layer (SBL) in both vertical and horizontal directions is the same as the concentration distribution in the horizontal direction in the convective boundary layer (CBL) [35], [36]. In

addition to the main AERMOD processor, this model consists of a meteorological preprocessor called AERMET and a geological preprocessor called AERMAP. In this research, ASTER satellite information was used as AERMAP input data. The output of this preprocessor includes the coordinates of each receiver and its height above sea level, the height of the roughness at the location of the receiver, and the geographical coordinates of the source. Due to the effect of unevenness in the ground and obstacles such as mountains, etc. on the distribution of pollutants, in this research due to the presence of complex topography from the point of view of air pollution, the effects of the slope of this area have been considered in the geographical information.

To run the AERMET preprocessor, the required data was first extracted and their quality evaluated and the upper and lower limit values of each parameter were controlled. In the next step, the data was organized for 24 hours using a storage interface file. Finally, two outputs were obtained from this pre-processor, one of which contained parameters of the atmospheric boundary layer such as wind friction speed, mixing height, length of the Monin Obukhov, and temperature and wind base height. Another output included vertical wind profile and temperature and deviation and wind component criteria. In this project, the amount of rainfall, cloud cover, measured pressure, and pressure relative to the surface of the free sea are considered as surface parameters, as well as dew point temperature, temperature, wind direction, wind speed, and humidity percentage as profile parameters. For this part of the modeling, the quality control and registration data of the National Meteorological Organization for the station located in Asaluyeh, which is located at 27 degrees and 48 minutes north latitude and 52 degrees and 61 minutes east longitude, has been used.

In this research, to cover all the sources in the scope of modeling studies and to focus on the desired source, as well as to include a range that can express the topographical and meteorological features of the region, the receivers in two separate network systems to the model have been introduced. Network receivers are defined in Cartesian coordinates and the desired monitoring station positions are introduced as separate receivers in the model. The arrangement of all receivers centered on a chimney has been chosen in such a way that it covers all sources and can express atmospheric phenomena on a medium and small scale, as well as the effects of topography and land use. Modeling of the distribution of pollutants coming out of the chimney is done for different time averages.

The AERMOD model requests specific information for each type of pollutant source. Chimneys used in this project were considered as a point source. Information such as pollutant emission rate, the height of pollutant release from the ground, temperature and speed of exhaust gas from the chimney, as well as the internal diameter of the chimney at the outlet were used as required information.

2.4. Modeling the distribution of pollutants by the interpolation method

Interpolation is the process of converting discrete data into continuous data. With this method, we can predict unknown values ​​through point or line-sampled data [29], [37]. In this study, interpolation was done by the KRIGING method using ArcGIS software. In the preparation of scatter maps, the KRIGING method is one of the most accurate models. The method used for interpolation was chosen because it has no systematic error and minimum variance in estimations. However, despite this, it has limitations in the preparation of the above maps. Among the most significant of these features is its absoluteness, which has caused the mismatch of the results of this model with the spatial distribution of geographical phenomena. At the same time, this feature enables geographical analysis in line with trend findings [32], [37]. An influential feature of the Kriging method is the absolute estimation of interpolation[38], [39]. Accordingly, the quantity estimation value at the sampling points is equal to the measured value, and the estimation variance is zero. Because of this feature, the kriging estimator doesn't cross the maximum sampling points when drawing lines of equal value. It also tends not to close off the range of interest when it crosses the boundary. This model minimizes the variance in estimating the unknown quantity of known points or coordinates.

2.5. Pollutants studied

This study investigated CO, NOX, and SOX pollutants, which are known to be the primary air pollutants caused by refinery activity. Pollutants were evaluated by collecting information about their origin, effects, permissible limits, etc. This information is displayed in the table1.

Table 1- introduction of investigated pollutants (CO, NOx, SOx)

| National standard and EPA limit in ppm (ug/m3) | Source | Health and environmental effects | Pollutant name |
| --- | --- | --- | --- |
| 1hour: 35(40000) 8 hours: 9(10000) | - Incomplete combustion of fossil fuels - Combustion of waste gases in the burner of refineries | - Cardiovascular diseases - Neurological diseases - Fibrin breakdown - Death at higher concentrations | CO |
| 1 hour: 0.1(200) 1 year : 0.053(100) | - Consumption of fossil fuels in power plants - Combustion of fuel in stationary and mobile sources | - Production of nitric acid and severe corrosion of metals - Create smog - Greenhouse Effect - Production of acid rain | NOx |
| 1 hour: 0.075 (196) 24 hours: 0.14 (395) | - Burning fossil fuels - Metal purification processes | - Damage to the respiratory system - Decreased plant growth - Creation of smog | SOx |

2.6.Environmental Assessment

Simulation of the data led to the output of the number of pollutants at each point of the study area. The map overlay method was used to display these results. This method is done by matching transparent maps with each other, and each map is evaluated for a certain geographical area. Using this method is a smart idea for projects whose scope of activity spans a wide area, as it was in this study. Simulation results were compared with the allowed amount of these pollutants in national and international standards in order to evaluate the calculated concentrations.

3. Results

In Figure 3, the wind rose in the area can be seen based on the information obtained from the national weather station located in the Asaluyeh area about the directions and speeds of the winds in the area. To create a visual understanding of the speed and direction of the winds in the wind rose area, a drawing was drawn on the map of the natural features of the studied area. According

to the speed of the winds in the region, these winds are grouped into several categories. The frequency of occurrence of winds in each group can be observed in the figure below. The horizontal axis represents the classification of winds, and the vertical axis shows the frequency percentage.

Figure 3- show the wind rose of region on the map and The state of distribution of the prevailing winds in the region according to speed

To ensure accuracy of the sampling results, the statistical analysis of the Q-Q chart is shown in Figure 4. In Figure 5, each pollutant distribution rate is separately shown. In the displayed results, chimneys and burners at the refinery site are considered sources of air pollution. According to the map, the red line represents Siraf city, while the blue line represents the study area. Based on the fact that each of the investigated pollutants has two standard concentration limits, the output was taken for each of the pollutants in each of the time intervals included in the standard.

Figure 4- The results of validation of sampling data in the form of Q-Q plot a) analysis of CO samples b) analysis of NO2 samples c) analysis of SO2 samples

In Figure 5, each pollutant distribution rate is separately shown. In the displayed results, chimneys and burners at the refinery site are considered sources of air pollution. According to the map, the red line represents Siraf city, while the blue line represents the study area. Based on the fact that each of the investigated pollutants has two standard concentration limits, the output was taken for each of the pollutants in each of the time intervals included in the standard. In these results, the concentration of pollutants and the way they spread are displayed using ranges with different colors for different concentration ranges. The results of interpolation using the KRIGING method are shown separately for each pollutant in figure 6. Based on the data obtained from sampling each pollutant, these results were obtained. An interpolated concentration of each pollutant in the study area is displayed using color spectra based on the concentration at the sampling site. In this paper, the interpolation results for CO pollutants are presented using the eight-hour limit, those for NO2 pollutants are presented using the one-year limit, and those for SO2 pollutants are presented using the 24-hour limit.

f

e

d

c

b

a

Figure 5-Showing the results of simulating the concentration and distribution of pollutants in AERMOD. a) CO in an average time of  8-hours b)CO in average time of 1-hour c)NO2 in an average of 1-hour d)NO2 in an average of  1-year e)SO2 in an average of 1-hour

Figure 6 shows the results of interpolation from ArcGIS using the KRIGING method for each pollutant. These results have been obtained by analyzing the data obtained from sampling each pollutant. Using color spectra, the interpolated concentration of each pollutant within the study area is displayed as a function of the concentration of each pollutant at the sampling site. According to the interpolation results, CO pollutants are interpolated based on the eight-hour limit, NO2 pollutants are interpolated based on the one-year limit, and SO2 pollutants are interpolated based on the 24-hour limit.

c

b

a

Figure 6- The results of analyzing the concentration and distribution of pollutants by interpolation using KRIGING method a) CO pollutant b) NOx pollutant) SOx pollutan

4.Discussion

The shape of wind rose shows that the wind in this area mainly blows from northwest to southeast. As can be seen in Figure 3, wind speeds between 2.1 and 0.5 m/s have the highest occurrence frequency in the region, while wind speeds above 11 m/s in the region are very insignificant and amount to 0.3%. In the preliminary studies of the air quality of the Siraf city area, it can be stated that this study area is restricted by the Zagros Mountain range, and on the other hand, according to the drawn wind direction, it can

be seen that the direction of the winds coming from the refineries located in Asaluyeh region, they blow towards the city of Siraf in the same direction as the prevailing winds, i.e. northwest to southeast. By observing the shape of the flood drawn on the Asaluyeh area and before running the simulation and obtaining the manner and extent of the spread and distribution of the pollutants in the Asaluyeh area and the extent of the pollution of the area as well as the privacy of the city of Siraf from the activity of its refineries, it can be said that this Two factors, i.e. the direction of the prevailing winds in the Asaluyeh area and the limitation of the study area of ​​the new city of Siraf on one side by the extension of the Zagros mountain range, can potentially be the reasons for the influence of the area of ​​the new city of Siraf from the activity of Asaluyeh refineries.

4.1. Carbon monoxide pollutant results

The results of the one-hour carbon monoxide pollutant concentration modeling results can be seen in Figure 5-b. As can be seen in this image, the maximum concentration of this pollutant in the modeling range is equal to 2428 μg/m3, which is lower than the free air standard of this pollutant for exposure for one hour, which is equal to 40000 μg/m3. The occurrence of the highest concentrations in the area of ​​the refinery and its radial distribution according to the height of the polluting sources from the ground surface shows the correct implementation of the simulation. It can also be seen that the concentration of this pollutant in the city of Siraf and its surrounding area is within the standard limit during the modeling period. The results of the 8-hours carbon monoxide pollutant concentration modeling results can be seen in Figure 5-a. As can be seen in this image, the maximum concentration of this pollutant in the modeling range is estimated to be 418 μg/m3, which is compared to the standard of free air carbon monoxide pollutant for exposure for eight hours, which is equal to 10,000 μg/m3. It is much less. It is noteworthy that this maximum concentration value is also in the same vicinity of pollutant-producing sources. A high concentration of this pollutant can be seen in the areas of the Zagros Wedge range. It seems that in this range the software has modeled a stable state of flow in the atmospheric layers and as a result, an unusual concentration has been displayed, which is a minor calculation error. It can be ignored. It can also be seen that the study area of ​​Siraf city and its surrounding area has a concentration within the standard limit in the period when the modeling was done, including the presence of high relative humidity in the area, effectively reducing the spread of pollutants and affecting the stability of the pollutant. It has resulted in certain areas. Comparing the results of interpolation and simulation in 8-hour exposure shows high compatibility between the results. In both exits, the maximum pollution can be seen in the refinery area and the main direction of the emission of pollutants towards the coast. On the other hand, in both models, the southeastern areas of the Siraf range have the maximum concentrations in the study area. If we compare the distribution and concentration of pollutants in continuous areas, the results are very similar, but it should be noted that the results obtained for There is a difference in the maximum pollutant concentration using the interpolation and simulation method. So that the maximum concentration in the interpolation method is 7.2 PPM and, in the simulation, it is about 3.7 PPM. It seems that the reason for this difference is due to the way of calculations in the simulation. The software analyzes the pollutant concentration using an extended range, while in the interpolation method, the concentration is checked as a point and discrete and finally integrated, so in certain points such as the vicinity of the chimney, instant high concentrations lead to the difference between the results.

4.2. Nitrogen dioxide pollutant result

The existing standard for this gas is an annual and one-hour standard. The results of modeling the spread and distribution of annual concentration of nitrogen dioxide pollutants can be seen in figure 5-d. As can be seen in this image, the maximum concentration of this pollutant in the study area is estimated to be 210 μg/m3, which is compared to the free air standard of nitrogen dioxide pollutant for the annual exposure period, which is equal to 100 μg/m3 in some Areas are exceeded. However, according to picture 5-d, it can be seen that the limits and boundaries of the siraf city are in the permitted part of this pollutant. In other words, the modeling shows that the concentrations of this gas in the limits and boundaries of Siraf city are completely below the standard level. In such a way that the maximum modeled concentration in this area is below 100 μg/m3. However, the high level of this pollutant in the area of ​​the refinery has long-term health risks for the employees. To further investigate the amount of NO2 for  1-hour, it was also modeled, the result of which can be seen in picture 5-c. Based on this picture, the maximum concentration observed in the whole picture is below 62 μg/m3, which is in the same range as Asaluyeh. However, the maximum concentration observed in the limits of Siraf city is below 5 μg/m3. The low number of this number indicates that this pollutant is below the standard limit. In general, it can be seen that the spread of NO2 pollutants is such that in addition to the refinery area, the surrounding areas of Siraf city are also involved in high pollution. The comparison between interpolation and simulation results for NO2 pollutants shows that there is a match between the results. In both results, the distribution of pollutants with a concentration higher than the standard limit is observed in the western areas of Siraf city. It should also be mentioned that in both cases, the amount of pollutants investigated in Siraf city is lower than the standard limit. The maximum amount of pollutant in both outputs has values ​​close to each other. It is worth mentioning that in the results of the simulation, a radial pollutant diffusion pattern is seen, while in the results of the interpolation, a linear-transverse pattern is seen. The reason for this can be because in the KRIGING method for an area variable with a spatial structure, the distribution is such that the similarity of the area variable value is greater for points close to each other than points far from each other, and therefore with increasing distance The space or time between the samples reaches such a point that from then on, the variable value of the area in the points around each other will not have much effect on each other, and it is called the range distance or radius of influence. The larger impact range implies a wider spatial structure. In this case and according to the given explanation, the diffusion pattern is separated from the radial mode and becomes linear. However, the comparison of the area range between the simulation and interpolation results despite the difference in the diffusion patterns. It

shows that there is a high matching between high and low-density areas of the pollutant. Therefore, the existence of matching between the concentrations of pollutants in different areas and the approximate matching of the areas affected by the pollutant indicates the correct simulation and the reliability of the results of the simulation by AERMOD.

4.3. Sulfur dioxide pollutant result

The results of 24-hours sulfur dioxide pollutant concentration modeling results can be seen in picture 5-f. As can be seen in this figure, the maximum concentration of this pollutant in the modeled range of the image is estimated to be 163 μg/m3, which is compared to the free-air standard of sulfur dioxide pollutant for a 24-hours exposure period, which is 365 μgr/m3. As can be seen, there is a concentration that is below the permissible limit in the city and surrounding areas of Siraf. In other words, the maximum concentration observed in the area and boundaries of Siraf city is about 80 μg/m3. Pollution concentrations suddenly decreased near the sources of pollution, which suggests an immediate distribution of this pollutant, which, in turn, led to a low concentration of pollutants in Siraf city. The results of modeling the spread and distribution of the annual concentration of sulfur dioxide pollutant can be seen in figure 5-e. As can be seen in this figure, the maximum concentration of this pollutant in the modeled range of the image is estimated to be equal to 37.7 μg/m3, which is compared to the free-air standard of sulfur dioxide-oxide pollutant for the year, which is equal to 80 μg/m3. Which has a value lower than the permissible amount of exposure. In this model, pollutant distribution can be confirmed. The comparison between the results of interpolation and simulation of SO2 pollutant distribution between the refinery plant and the study area of ​​Siraf city shows high compatibility between the amount and direction of pollutant emission. However, it seems that in the simulation results, pollutant distribution occurs at closer distances to pollutant sources, while in the model obtained from interpolation, distribution occurs in more distant areas. It seems that the occurrence of this difference in the results is due to the presence of minor errors in the data related to the parameters of the boundary layer in the input data to the AERMOD software. Also, the difference between the linear and radial emission pattern can be seen, such as NO2 pollutant, which has a limited effect on determining the pollution ranges. The comparison between the output results from interpolation and simulation shows the compatibility of both methods and the reliability of the simulation results.

5. Conclusion

The emission of air pollutants by various industries causes air pollution. The pollutants released by these industries cause environmental and health risks in nearby urban areas. In this study, the emission of SO2, NO2, and CO pollutants from a refinery site in the South Pars energy region and its effect on the pollution situation of Siraf city were investigated. The release of pollutants was done using two methods of kriging interpolation and simulation with AERMOD software. The results determined the concentration of the investigated pollutants in each point of the study area as well as their distribution method. Also, to measure the accuracy of the results, a comparison was made between the results of interpolation and simulation. In the study area, prevailing winds blow from the northwest to the southeast. And the highest frequency belongs to winds with a speed of 0.5 to 2.1 m/s. The topography of the area and the direction of the winds were identified as two natural factors influencing the spread of pollutants and increasing their concentration in the Siraf area. The maximum concentration of carbon monoxide pollutant for 1 hour exposure was 2428 µg/m3. This amount is lower than the maximum allowed pollutant concentration (40000 µg/m3). The maximum concentration of this pollutant in eight-hour exposure was 418 µg/m3, which is within the standard permissible range. A comparison of emission patterns and concentration of carbon monoxide pollutants in two methods of interpolation and simulation shows that there is an agreement between their results. Comparing the results obtained with previous studies such as the study of Sabah Abdul-Wahab and colleagues [40] shows the similarity between the diffusion patterns. The radial emission pattern of carbon monoxide pollutants was seen as the main pattern in this study and other similar studies [41], [42], [43]. The maximum concentration of nitrogen dioxide pollutants was 62 µg/m3 in one-hour exposure and 210 µg/m3 in annual exposure. The one-year exposure concentration is higher than the standard limit (100 µg/m3). Areas with dense buildings and high barriers have the highest concentrations. The comparison between interpolation and simulation results for nitrogen dioxide pollutants shows the matching between concentrations. Differences can be seen between the diffusion patterns, the cause of which is related to the special way of performing calculations in the kriging interpolation method. The emission patterns obtained in the simulation are consistent with the results of the studies of Xueying Zhang et al [44]. The results of interpolation and linear patterns obtained in the studies of Mark D. Gibson and colleagues [45] have also been considered. The maximum concentration of sulfur oxide pollutant was 163 µg/m3 in 24-hour exposure and 37.7 µg/m3 in annual exposure. The obtained concentrations are within the permissible range of the standard. Between the results of interpolation and simulation, matching is observed in the amount of pollutant concentration, but there are differences between the emission patterns. The results obtained in terms of pollutant diffusion pattern and concentration distribution are compatible with the results obtained in the study of Farzaneh Taksibi et al [46]. and the study of Ezgi Akyuz et al [47]. This study shows the high level of NO2 pollutants in the Siraf urban area as a result of emissions from industrial pollutant sources. The amount of SO2 pollutants is also significant. The concentration of CO pollutants is in a suitable range. Comparing the results between interpolation and simulation shows the compatibility between the results and the reliability of the results obtained from the simulation with AERMOD software. The results of these studies can be used to investigate the effects of control measures, health risk assessments, and environmental conditions in the region.

References

[1]	S. S. Babatola, ‘Globalattributableburdento ofairdiseasespollution’, J Public Health Afr, vol. 9, no. 3, Dec. 2018, doi: 10.4081/jphia.2018.813.

[2]	G. Wu, L. C. Liu, Z. Y. Han, and Y. M. Wei, ‘Climate protection and China’s energy security: Win–win or tradeoff’, Appl Energy, vol. 97, pp. 157–163, Sep. 2012, doi: 10.1016/J.APENERGY.2011.11.061.

[3]	J. G. Williamson, ‘Migrant Selectivity, Urbanization, and Industrial Revolutions’, 1988. [Online]. Available: https://www.jstor.org/stable/1973573

[4]	A. Rauf, J. Zhang, J. Li, and W. Amin, ‘Structural changes, energy consumption and carbon emissions in China: Empirical evidence from ARDL bound testing model’, Structural Change and Economic Dynamics, vol. 47, pp. 194–206, Dec. 2018, doi: 10.1016/J.STRUECO.2018.08.010.

[5]	K. Nansai et al., ‘Consumption in the G20 nations causes particulate air pollution resulting in two million premature deaths annually’, Nature Communications 2021 12:1, vol. 12, no. 1, pp. 1–12, Nov. 2021, doi: 10.1038/s41467-021-26348-y.

[6]	F. M. Adebiyi, ‘Air quality and management in petroleum refining industry: A review’, Environmental Chemistry and Ecotoxicology, vol. 4, pp. 89–96, Jan. 2022, doi: 10.1016/J.ENCECO.2022.02.001.

[7]	A. de Agostini, P. Cortis, and A. Cogoni, ‘Monitoring of air pollution by moss bags around an oil refinery: A critical evaluation over 16 years’, Atmosphere (Basel), vol. 11, no. 3, Mar. 2020, doi: 10.3390/atmos11030272.

[8]	L. O. Shtripling and V. v. Bazhenov, ‘Emission process system organisation of pollutants into the atmosphere for refinery enterprises’, in Procedia Engineering, Elsevier Ltd, 2015, pp. 349–356. doi: 10.1016/j.proeng.2015.07.280.

[9]	A. J. Cohen et al., ‘Estimates and 25-year trends of the global burden of disease attributable to ambient air pollution: an analysis of data from the Global Burden of Diseases Study 2015’, The Lancet, vol. 389, no. 10082, pp. 1907–1918, May 2017, doi: 10.1016/S0140-6736(17)30505-6.

[10]	B. S. Fakinle, E. L. Odekanle, A. P. Olalekan, H. E. Ije, D. O. Oke, and J. A. Sonibare, ‘Air pollutant emissions by anthropogenic combustion processes in Lagos, Nigeria’, Cogent Eng, vol. 7, no. 1, Jan. 2020, doi: 10.1080/23311916.2020.1808285.

[11]	V. Todorov and I. Dimov, ‘Innovative Digital Stochastic Methods for Multidimensional Sensitivity Analysis in Air Pollution Modelling’, Mathematics 2022, Vol. 10, Page 2146, vol. 10, no. 12, p. 2146, Jun. 2022, doi: 10.3390/MATH10122146.

[12]	S. Chen, P. Oliva, and P. Zhang, ‘The effect of air pollution on migration: Evidence from China’, J Dev Econ, vol. 156, p. 102833, May 2022, doi: 10.1016/J.JDEVECO.2022.102833.

[13]	X. Xu, H. Yang, and C. Li, ‘Theoretical Model and Actual Characteristics of Air Pollution Affecting Health Cost: A Review’, International Journal of Environmental Research and Public Health 2022, Vol. 19, Page 3532, vol. 19, no. 6, p. 3532, Mar. 2022, doi: 10.3390/IJERPH19063532.

[14]	H. Kanaboshi, F. Sano, J. Oda, K. Akimoto, and N. Onishi, ‘Cost-efficient measures in the oil refinery and petrochemical sectors for the reduction of CO2 emissions under the Paris Agreement and air pollution under the MARPOL Convention’, Energy and Climate Change, vol. 2, p. 100027, Dec. 2021, doi: 10.1016/J.EGYCC.2021.100027.

[15]	D. Lv, S. Lu, X. Tan, M. Shao, S. Xie, and L. Wang, ‘Source profiles, emission factors and associated contributions to secondary pollution of volatile organic compounds (VOCs) emitted from a local petroleum refinery in Shandong’, Environmental Pollution, vol. 274, p. 116589, Apr. 2021, doi: 10.1016/J.ENVPOL.2021.116589.

[16]	H. Lux et al., ‘Outdoor air pollution from industrial chemicals causing new onset of asthma or COPD: a systematic review protocol’, Journal of Occupational Medicine and Toxicology, vol. 15, no. 1, Dec. 2020, doi: 10.1186/s12995-020-00289-6.

[17]	J. Huang, J. Shen, L. Miao, and W. Zhang, ‘The effects of emission trading scheme on industrial output and air pollution emissions under city heterogeneity in China’, J Clean Prod, vol. 315, Sep. 2021, doi: 10.1016/j.jclepro.2021.128260.

[18]	J. Wu, Y. Jia, M. Cheng, and X. Xia, ‘A complex network perspective on embodiment of air pollutants from global oil refining industry’, Science of The Total Environment, vol. 824, p. 153740, Jun. 2022, doi: 10.1016/J.SCITOTENV.2022.153740.

[19]	M. Kampa and E. Castanas, ‘Human health effects of air pollution’, Environmental Pollution, vol. 151, no. 2, pp. 362–367, Jan. 2008, doi: 10.1016/J.ENVPOL.2007.06.012.

[20]	A. Mohammadiha, H. Malakooti, and V. Esfahanian, ‘Development of reduction scenarios for criteria air pollutants emission in Tehran Traffic Sector, Iran’, Science of the Total Environment, vol. 622–623, pp. 17–28, May 2018, doi: 10.1016/j.scitotenv.2017.11.312.

[21]	L. Bai, C. Li, C. W. Yu, and Z. He, ‘Air pollution and health risk assessment in Northeastern China: A case study of Jilin Province’, https://doi.org/10.1177/1420326X20979274, vol. 30, no. 10, pp. 1857–1874, Jan. 2021, doi: 10.1177/1420326X20979274.

[22]	L. Zeng et al., ‘A circulating ventilation system to concentrate pollutants and reduce exhaust volumes: Case studies with experiments and numerical simulation for the rubber refining process’, Journal of Building Engineering, vol. 35, p. 101984, Mar. 2021, doi: 10.1016/J.JOBE.2020.101984.

[23]	P. Ghobadi and N. Nasrollahi, ‘Assessment of pollutant dispersion in deep street canyons under different source positions: Numerical simulation’, Urban Clim, vol. 40, p. 101027, Dec. 2021, doi: 10.1016/J.UCLIM.2021.101027.

[24]	J. Liu, O. Duru, and A. W. K. Law, ‘Assessment of atmospheric pollutant emissions with maritime energy strategies using bayesian simulations and time series forecasting’, Environmental Pollution, vol. 270, p. 116068, Feb. 2021, doi: 10.1016/J.ENVPOL.2020.116068.

[25]	A. Daly and P. Zannetti, ‘Air Pollution Modeling-An Overview’, 2007. [Online]. Available: http://www.arabschool.org.sy

[26]	A. Mott and H. Guo, ‘Odour dispersion modelling, impact criteria, and setback distances for an oil refinery plant’, Atmos Environ, vol. 270, p. 118879, Feb. 2022, doi: 10.1016/J.ATMOSENV.2021.118879.

[27]	D. J. Pesic, M. D. Blagojevic, and S. M. Glisovic, ‘The model of air pollution generated by fire chemical accident in an urban street canyon’, Transp Res D Transp Environ, vol. 16, no. 4, pp. 321–326, 2011, doi: 10.1016/j.trd.2011.01.012.

[28]	A. Munjal, S. Sharma, A. K. Nema, and S. H. Kota, ‘Factors affecting particulate matter levels near highway toll plazas in India’, Transp Res D Transp Environ, vol. 110, p. 103403, Sep. 2022, doi: 10.1016/J.TRD.2022.103403.

[29]	C. Xu, J. Wang, M. Hu, and W. Wang, ‘A new method for interpolation of missing air quality data at monitor stations’, Environ Int, vol. 169, p. 107538, Nov. 2022, doi: 10.1016/J.ENVINT.2022.107538.

[30]	P. C. Chen and Y. T. Lin, ‘Exposure assessment of PM2.5 using smart spatial interpolation on regulatory air quality stations with clustering of densely-deployed microsensors’, Environmental Pollution, vol. 292, p. 118401, Jan. 2022, doi: 10.1016/J.ENVPOL.2021.118401.

[31]	A. M. Talib and Dr. M. N. Jasim, ‘GIS-GPS based national air pollution monitoring system’, Mater Today Proc, Jun. 2021, doi: 10.1016/j.matpr.2021.05.445.

[32]	A. Kumar, S. Dhakhwa, and A. K. Dikshit, ‘Comparative Evaluation of Fitness of Interpolation Techniques of ArcGIS Using Leave-One-Out Scheme for Air Quality Mapping’, Journal of Geovisualization and Spatial Analysis, vol. 6, no. 1, p. 9, 2022, doi: 10.1007/s41651-022-00102-4.

[33]	L. Han, J. Zhao, and Z. Gu, ‘Assessing air quality changes in heavily polluted cities during the COVID-19 pandemic: A case study in Xi’an, China’, Sustain Cities Soc, vol. 70, p. 102934, 2021, doi: https://doi.org/10.1016/j.scs.2021.102934.

[34]	S. S. Mousavi, G. Goudarzi, S. Sabzalipour, M. M. Rouzbahani, and E. Mobarak Hassan, ‘An evaluation of CO, CO2, and SO2 emissions during continuous and non-continuous operation in a gas refinery using the AERMOD’, Environmental Science and Pollution Research, vol. 28, no. 40, pp. 56996–57008, Oct. 2021, doi: 10.1007/s11356-021-14493-2.

[35]	A. J. Cimorelli et al., ‘AERMOD: A Dispersion Model for Industrial Source Applications. Part I: General Model Formulation and Boundary Layer Characterization’, J Appl Meteorol Climatol, vol. 44, no. 5, pp. 682–693, May 2005, doi: 10.1175/JAM2227.1.

[36]	D. J. Naghan, A. Mahmoodi, A. Tavasolifar, M. S. Saeidi, and Y. Jalilpoor, ‘Analyzing the health risk assessment of particles in Isfahan steel company by AERMOD model’, 2021. [Online]. Available: http://japh.tums.ac.ir

[37]	R. Esmaili Climatology, M. Mohammadi, A. Professor, in Environmental Biotechnology, and K. Motahar, ‘Providing the best pattern Interpolation in the spatial distribution of partiulate matter smaller than 2.5 microns of mashhad’, Journal of Research in Environmental Health, vol. 7, no. 3, pp. 201–215, Nov. 2021, doi: 10.22038/JREH.2021.60400.1446.

[38]	C. Liu et al., ‘Optimal estimation of initial concentrations and emission sources with 4D-Var for air pollution prediction in a 2D transport model’, Science of the Total Environment, vol. 773, Jun. 2021, doi: 10.1016/j.scitotenv.2021.145580.

[39]	S. H. Saleh and A. F. Hassoon, ‘Atmospheric stability classes and its effect on co concentration emission around Kirkuk refinery’, in Journal of Physics: Conference Series, IOP Publishing Ltd, Jul. 2021. doi: 10.1088/1742-6596/1963/1/012034.

[40]	S. Abdul-Wahab, S. Fadlallah, and M. Al-Rashdi, ‘Evaluation of the impact of ground-level concentrations of SO2, NOx, CO, and PM10 emitted from a steel melting plant on Muscat, Oman’, Sustain Cities Soc, vol. 38, pp. 675–683, 2018, doi: https://doi.org/10.1016/j.scs.2018.01.048.

[41]	P. Amoatey, H. Omidvarborna, M. S. Baawain, and A. Al-Mamun, ‘Emissions and exposure assessments of SOX, NOX, PM10/2.5 and trace metals from oil industries: A review study (2000–2018)’, Process Safety and Environmental Protection, vol. 123, pp. 215–228, 2019, doi: https://doi.org/10.1016/j.psep.2019.01.014.

[42]	D. Huang and H. Guo, ‘Dispersion modeling of odour, gases, and respirable dust using AERMOD for poultry and dairy barns in the Canadian Prairies’, Science of The Total Environment, vol. 690, pp. 620–628, 2019, doi: https://doi.org/10.1016/j.scitotenv.2019.07.010.

[43]	R. A. Myshko and N. I. Kurakina, ‘GIS for Assessment and Modeling Air Pollution by Industrial Facilities’, Proceedings of the 2021 IEEE Conference of Russian Young Researchers in Electrical and Electronic Engineering, ElConRus 2021, pp. 1789–1802, Jan. 2021, doi: 10.1109/ELCONRUS51938.2021.9396270.

[44]	X. Zhang et al., ‘A hybrid approach to predict daily NO2 concentrations at city block scale’, Science of The Total Environment, vol. 761, p. 143279, 2021, doi: https://doi.org/10.1016/j.scitotenv.2020.143279.

[45]	M. D. Gibson, S. Kundu, and M. Satish, ‘Dispersion model evaluation of PM2.5, NOx and SO2 from point and major line sources in Nova Scotia, Canada using AERMOD Gaussian plume air dispersion model’, Atmos Pollut Res, vol. 4, no. 2, pp. 157–167, 2013, doi: https://doi.org/10.5094/APR.2013.016.

[46]	F. Taksibi, H. Khajehpour, and Y. Saboohi, ‘On the environmental effectiveness analysis of energy policies: A case study of air pollution in the megacity of Tehran’, Science of The Total Environment, vol. 705, p. 135824, 2020, doi: https://doi.org/10.1016/j.scitotenv.2019.135824.

[47]	E. Akyuz and B. Kaynak, ‘Use of dispersion model and satellite SO2 retrievals for environmental impact assessment of coal-fired power plants’, Science of The Total Environment, vol. 689, pp. 808–819, 2019, doi: https://doi.org/10.1016/j.scitotenv.2019.06.464.