# Safety Distance Assessment for Vapour Cloud Explosion in the PSA Unit of Abadan Refinery: Comparison of TNT Equivalency, BST, and TNO Multi-Energy Blast Models

Seyedeh Atefeh Hosseini1*

Shahid Tondgoyan Faculty, Petroleum University of Technology, Abadan, Iran

Abdolrahim Taheri2

Shahid Tondgoyan Faculty, Petroleum University of Technology, Abadan, Iran

Mohsen Motevassel3

Shahid Tondgoyan Faculty, Petroleum University of Technology, Abadan, Iran

Alireza Jafari4

Abadan Oil Refining Company, Abadan, Iran

Abstract

Vapour cloud explosion (VCE) modelling plays a critical role in defining safety distances for congested

gas-processing units, yet predicted consequences remain highly sensitive to the selected blast model. This study presents a consequence-based safety distance assessment for a hydrogen PSA unit at Abadan Refinery using PHAST 8.4, comparing three widely applied approaches: TNT equivalency, Baker–Strehlow–Tang (BST), and TNO Multi-Energy (ME). Representative loss-of-containment scenarios were simulated to estimate flammable cloud formation (tracked at the mixture LFL of 5.1 vol% ≈ 51,000 ppm) and subsequent VCE overpressure footprints. Safety distances were interpreted according to GS EP SAF 253 fire-zone criteria, with 200 mbar (0.2 bar) adopted as the governing explosion threshold.
Results demonstrate pronounced model dependence. For the 20 mm leak case, the 0.2 bar radius varied from 1.72 m (ME) to 3.84 m (BST), while TNT predictions ranged from 0.88 to 1.89 m depending on the assumed yield (1–10%). Divergence increased at lower overpressure levels relevant to escalation and siting decisions. The findings highlight the limitations of unqualified TNT use and support an envelope-based approach for safety distance definition, in which model assumptions are reported transparently and conservative distances are selected consistent with congestion, confinement, and applicable standards.

Keywords: vapour cloud explosion, safety distance, PHAST 8.4, PSA unit, TNT equivalency, BST, TNO Multi‑Energy, GS EP SAF 253

1. Introduction

The PSA (Pressure Swing Adsorption) unit contains pressurised gas inventories and dense equipment layouts that can support the formation of flammable clouds following loss of containment[1,4,12]. If a delayed ignition occurs, rapid flame acceleration may generate damaging overpressures[13]. For layout screening and emergency planning, practitioners often translate consequence predictions into ‘safety distances’ or ‘fire‑zone’ boundaries [3,5]. However, predicted distances can vary substantially with the chosen blast model and the assumptions used to represent congestion, confinement, and fuel reactivity [6,7].

Despite extensive use of TNT, BST, and Multi-Energy models in consequence analysis, limited published work explicitly quantifies their divergence for hydrogen PSA units under identical dispersion and damage criteria [4]. This gap complicates model selection for layout screening in existing refineries, where conservative but realistic safety distances are required [1,3,12].

This paper develops a consistent modeling workflow in PHAST 8.4 for VCE outcome assessment [2] and compares (i) TNT equivalent, (ii) Baker-Strehlow-Tang (BST) methods, and (iii) TNO Multi-Energy (ME) methods [6,7,11]. The output is used to define safety distances according to GS EP SAF 253 criteria [3], focusing on an overpressure threshold of 0.2 bar (200 mbar).

The novelty of this work lies in the consistent comparison of TNT, Baker-Strehlow-Tang, and TNO multi-energy explosion equivalent models for a hydrogen PSA unit under the same dispersion basis and damage criterion. Unlike previous studies, seasonal meteorological data are considered for Abadan, while atmospheric stability is kept constant to isolate seasonal effects. The results are directly interpreted within the GS EP SAF 253 fire zone framework and provide practical guidance for selecting conservative safety distances in dense refinery units.

2. Case Study and Input Basis

The case study focuses on the PSA (Pressure Swing Adsorption) unit of Abadan Refinery, Iran. The unit handles hydrogen-rich process gas under pressure and is characterised by relatively congested equipment arrangements typical of hydrogen recovery sections in large refining complexes [1,12]. The source item considered is a pressurised vessel located approximately 1.5 m above grade. To preserve confidentiality, detailed equipment identifiers are not disclosed; however, the selected scenario is representative of credible loss-of-containment events in PSA service.

Leak scenarios were defined to represent credible release sizes commonly used for consequence and risk screening, corresponding to equivalent circular hole diameters of 20, 50, and 80 mm [8]. For dispersion and fire-zone screening purposes, the effective release height was taken as 1 m above grade, consistent with standard assumptions for horizontal or impacted gas jets.

The released fluid was modelled as a water-saturated hydrogen-rich process gas. On a dry basis, the composition consisted of H₂ (73.76 mol%), CO (4.08 mol%), CO₂ (16.31 mol%), CH₄ (4.30 mol%), and N₂ (1.55 mol%), which is representative of PSA off-gas streams in refinery hydrogen management systems. Operating conditions were fixed at 22 bar and 40 °C to provide a consistent baseline for isolating the effects of leak diameter and meteorological conditions on flammable cloud formation (Table 1).

Table 1 Process gas composition and operating conditions used as PHAST inputs.

| Category | Parameter | Value |
| --- | --- | --- |
| Gas composition (mol%) | H2 | 73.76 |
| Gas composition (mol%) | CO | 4.08 |
| Gas composition (mol%) | CO2 | 16.31 |
| Gas composition (mol%) | CH4 | 4.30 |
| Gas composition (mol%) | N2 | 1.55 |
| Gas composition | Water content | Saturated |
| Operating conditions | Pressure | 22 bar |
| Operating conditions | Temperature | 40 °C |
| Release setup | Release type | Horizontal orifice leak (hole) |
| Release setup | Release elevation / height of interest | 1 m / 1 m |
| Release setup | Maximum release duration | 3600 s |

For dispersion analysis, the flammable criterion was defined as the mixture lower flammability limit (LFL) of 5.1 vol%, expressed in PHAST as 51,000 ppm [2,4]. Where relevant, a 10-minute initial release duration was applied to align with the GS EP SAF 253 fire-zone definition for unignited flammable gas cloud dispersion [3]. In addition, a maximum release duration of 3600 s was specified in PHAST to ensure that cloud development was not artificially constrained.

Seasonal ambient conditions were defined using daily meteorological records for Abadan during 2024. The data were aggregated to monthly averages and subsequently to representative seasonal values in order to reduce short-term variability while preserving systematic seasonal contrasts relevant to atmospheric dispersion. Seasonal temperature–relative humidity pairs were 26.57 °C / 43.97% (spring), 38.82 °C / 24.62% (summer), 28.08 °C / 40.54% (autumn), and 15.71 °C / 59.26% (winter). Corresponding seasonal mean wind speeds were 3.96, 4.74, 4.18, and 3.54 m/s, respectively.

Atmospheric stability was held constant at Pasquill class D for all seasons. Long-term observations indicate that neutral stability conditions dominate for a significant portion of the year in the Abadan region [9]. Maintaining a uniform stability class was therefore a deliberate choice to isolate the influence of seasonal variations in temperature, humidity, and wind speed on predicted flammable cloud mass, while avoiding confounding effects associated with changes in atmospheric stratification (Table 2).

Table 2 Seasonal meteorology inputs used in PHAST (Abadan, 2024).

| Season | Temperature (°C) | Relative humidity (%) | Wind speed (m/s) | Stability class |
| --- | --- | --- | --- | --- |
| Spring | 26.57 | 43.97 | 3.96 | D |
| Summer | 38.82 | 24.62 | 4.74 | D |
| Autumn | 28.08 | 40.54 | 4.18 | D |
| Winter | 15.71 | 59.26 | 3.54 | D |

This controlled case-study design provides a defensible basis for examining diameter season interactions and for determining whether the governing season defined by maximum flammable cloud mass and associated explosion  impact remains invariant or shifts with leak diameter, which is central to seasonal scenario selection in practical consequence analysis[10].

3. Methodology

3.1 Dispersion and Flammable Cloud Definition

Each leak case was simulated to obtain release rate history and the maximum flammable cloud footprint at the chosen LFL[2,4]. PHAST outputs used include mass release rate versus time, maximum concentration versus distance at specified height, and cloud footprint / maximum footprint. The flammable averaging time was maintained consistently across runs to avoid artefacts in cloud extent.

3.2 Vapour Cloud Explosion Modelling

The same flammable cloud basis was used as input to three explosion models: TNT equivalency, BST, and TNO Multi‑Energy[6,7]. Overpressure radii were extracted at multiple thresholds. In this study, the governing safety distance criterion follows GS EP SAF 253 and is taken as 200 mbar (0.2 bar) [3]. Additional thresholds (e.g., 0.14 and 0.05 bar) are reported for vulnerability screening and escalation assessment[11].

3.3 Safety Distance Interpretation (Fire‑Zone)

Safety distance is defined as the maximum downwind distance (or equivalent radius) at which the selected criterion is met. For VCE, the 0.2 bar radius is the primary fire‑zone boundary. To support design decisions, an envelope approach is applied: for each leak size, the maximum predicted radius among the credible models/assumptions is selected as the conservative safety distance.

4. Results and Discussion

4.1 Model Comparison for the 20 mm Leak Case

Table 3 summarises extracted overpressure radii for the 20 mm leak case from the three models. The TNO Multi‑Energy method predicts the smallest radii for the selected thresholds, while BST produces the most conservative results. TNT predictions depend strongly on the assumed yield (efficiency), highlighting the need to justify and report this parameter[13].

Table 3 Overpressure radii for the 20 mm leak case (extracted from PHAST runs).

| Model | Assumption | R = 0.20 bar (m) | R = 0.14 bar (m) | R = 0.05 bar (m) |
| --- | --- | --- | --- | --- |
| TNO Multi‑Energy | ME case (as modelled) | 1.72 | 2.30 | 5.64 |
| BST | High reactivity; medium obstacle density; flame expansion=3 | 3.84 | 5.26 | 12.65 |
| TNT equivalency | Yield=10% | 1.89 | 2.45 | 4.87 |
| TNT equivalency | Yield=5% | 1.50 | 1.94 | 3.86 |
| TNT equivalency | Yield=1% | 0.88 | 1.14 | 2.26 |

Figure 1 shows the overpressure decay with distance for the 20 mm leak case, comparing BST, TNT (yield = 10%), and TNO Multi-Energy models. The logarithmic scale highlights that BST predicts the highest overpressures at all distances, TNT is intermediate depending on the assumed yield, and multi-energy predicts the lowest overpressures. These differences are particularly significant at short distances (<50 m), which are critical for layout screening and fire-zone determination. At larger distances, model predictions converge, but the divergence at critical thresholds (0.2 bar, 0.14 bar, 0.05 bar) emphasizes the need for an envelope approach in defining conservative safety distances.

Figure 1 Overpressure vs. distance for the 20 mm leak scenario using BST, TNT, and TNO Multi-Energy models.

4.2 Implications for Safety Distance Selection

Using the GS EP SAF 253 criterion of 0.2 bar, the safety distance for the 20 mm case ranges from 0.88–1.89 m for TNT depending on yield, 1.72 m for Multi‑Energy, and 3.84 m for BST. Under an envelope approach, BST governs the conservative fire‑zone boundary when congestion/confinement justifies a high‑strength representation.

At lower overpressure thresholds (0.14 and 0.05 bar), model divergence becomes larger. This is important for siting decisions, as low overpressures can still affect lightweight structures, instrumentation, and secondary hazards [14,15].

It should be noted that the relatively small safety distances obtained in this study are primarily due to the availability of accessible materials with analytical and analytical sizes and the rapid dispersion characteristics of hydrogen-rich gases. Therefore, the results represent the design of local fire zone boundaries for screening, not plant siting distances in the larger region.

4.3 Expected Scaling Behaviour for Larger Leak Sizes (50, 80 mm)

For larger leak diameters (50 and 80 mm), the flammable cloud size and therefore the explosion energy potential generally increase, so larger safety distances are expected. The same workflow should be repeated for each leak size, ensuring that the dispersion basis (LFL threshold, release duration assumptions, and meteorology) is consistent. In practice, the final safety distance for the unit is often taken as the maximum distance among the analysed leak sizes at the governing criterion[4,8]. A detailed quantitative comparison for the 50 and 80 mm cases follows the same modelling logic and is omitted here for brevity.

5. Conclusions

A consistent PHAST 8.4 workflow was developed to estimate VCE safety distances for a PSA unit and to compare TNT equivalency, BST, and TNO Multi‑Energy models. The governing criterion for explosion fire‑zone definition was 200 mbar (0.2 bar) in accordance with GS EP SAF 253.

For the 20 mm case, BST predicted the largest overpressure radii (3.84 m at 0.2 bar and 12.65 m at 0.05 bar), Multi‑Energy predicted smaller radii (1.72 m at 0.2 bar and 5.64 m at 0.05 bar), and TNT results varied widely with yield (0.88–1.89 m at 0.2068 bar). Therefore, TNT should not be presented without a clearly justified yield assumption.

The study recommends an envelope approach: report model assumptions transparently, justify the model strength by congestion/confinement, and adopt the most conservative distance at the governing criterion as the safety distance used for layout screening and risk communication.

References

[1]  Lees, F. P., 2012. Lees’ Loss Prevention in the Process Industries, 4th ed., Elsevier, Oxford, UK.

[2]  DNV GL, 2019. PHAST 8.4 User Documentation, DNV GL Software, Høvik, Norway.

[3]  TotalEnergies, 2018. GS EP SAF 253: Fire Zones and Safety Distances, TotalEnergies Global Standard.

[4]  CCPS (Center for Chemical Process Safety), 2010. Guidelines for Chemical Process Quantitative Risk    Analysis, 2nd ed., AIChE, New York, USA.

[5]  Department of Defense, 2008. UFC 3-340-02: Structures to Resist the Effects of Accidental Explosions (formerly TM 5-1300), Washington, DC, USA.

[6]  TNO, 2005. Methods for the Calculation of Physical Effects (Yellow Book), 3rd ed., The Hague, The Netherlands.

[7] Baker, Q. A., Strehlow, R. A., and Tang, M. J., 1994. A model for prediction of vapour cloud explosion overpressures (BST method), in: Proceedings of the International Conference on Loss Prevention, Institution of Chemical Engineers, UK.

[8] IOGP, 2019. Risk Assessment Data Directory: Process Release Frequencies and Ignition Probabilities, Report 434-01, International Association of Oil & Gas Producers, London, UK.

[9] NOAA NCEI, 2025. Integrated Surface Database (ISD) – Global Hourly, National Centers for Environmental Information, Asheville, NC, USA.

[10]  Iowa State University, 2025. IEM MyWindrose: Windrose generation from station observations (OIAA Abadan), Iowa Environmental Mesonet, Ames, IA, USA.

[11]  van den Berg, A. C., 1985. The Multi-Energy Method: a framework for vapour cloud explosion blast prediction, Journal of Hazardous Materials, 12(1), pp. 1–10.

[12] Baker, Q. A., Cox, P. A., Kulesz, J. J., Strehlow, R. A., and Westine, P. S., 2012. Explosion Hazards and Evaluation, Elsevier, Amsterdam, The Netherlands.

[13] HSE (Health and Safety Executive), 2010. Vapour Cloud Explosions: Guidance for Consequence Assessment, Health and Safety Executive, UK.

[14] CCPS (Center for Chemical Process Safety), 1994. Guidelines for Evaluating the Characteristics of Vapor Cloud Explosions, Flash Fires, and BLEVEs, AIChE, New York, USA.

[15] NFPA, 2018. NFPA 68: Standard on Explosion Protection by Deflagration Venting, National Fire Protection Association, Quincy, MA, USA.