Title: The Effect of Environmental Factors on the Concentration of Fungal Bioaerosols Indoor a hospital: A Cross-Sectional Study

Ameneh Bahreini1, Kavoos Dindarloo2, Parvin Lakbala1*, Mahdi Behjati Ardakani3, Hamid Reza Ghaffari2

1-Social Determinants in Health Promotion Research Center, Hormozgan University of Medical Sciences, Bandar Abbas, Iran.

2-Department of Environmental Health Engineering, Faculty of Health, Hormozgan University of Medical Sciences, Bandar Abbas, Iran.

3-Department of Occupational Health Engineering Faculty of Health, Hormozgan Medical Sciences Bandar Abbas Iran.

Abstract

The present study was conducted to determine the effect of environmental factors on the concentration of fungal bioaerosols in a hospital. The air samples were collected using a one-stage Anderson sampler and a particle mass counter between September 2019 and February 2020. In total, 43 fungal samples were collected from the air of the wards and outdoors of a hospital in Jask City, Iran. Data were analyzed by t-test, one-way ANOVA, and Pearson correlation. The fungal species were found to be most numerous in this study were Penicillium (36.19%), Cladosperium (29.04%), and Aspergillus (21.90%). The highest and lowest densities of fungal bioaerosols were found in the internal ward and the infectious emergency ward, respectively. The results demonstrated a significant two-way correlation between temperature and fungal concentration (P=0.028). Additionally, a significant two-tailed correlation was found between the concentration of fungi in CFU/m3 and the number of companions accompanying the patients (P = 0.013). The highest and lowest densities of fungal bioaerosols were found in the internal ward and the infectious emergency ward, respectively. Managing the entry and exit of visitors and companions of patients to hospital wards as well as Installing an air purifier system is important in reducing airborne particles.

Keywords: Fungi, Airborne pollutant, environmental factors, Indoor, Hospitals,

Introduction

The air quality inside buildings, particularly hospitals, plays a crucial role in people's well-being and satisfaction (1). The primary cause of inadequate indoor air quality is linked to the presence of biological particles and airborne allergens, including bacteria, fungi, and viruses(2).  Viruses, bacteria, and fungi are among the causes of hospital-acquired infections, which cause health problems in treated patients and also cause hospital costs to increase. This is crucial for the recovery process of patients, as well as for the protection of visitors and healthcare professionals. Fungal contamination poses a significant threat to indoor air quality, especially in hospital buildings. Fungi, because of their opportunistic nature, can cause infections in individuals with compromised immune systems (3). Fungal spores, owing to their diminutive size, have a remarkable ability to disseminate across diverse hospital regions. The environment contains various types of fungi, and when people breathe in their spores, it can lead to allergic reactions, asthma, and respiratory infections. Fungal bioaerosols are resistant to environmental stress and airborne transport (4). Fungal spores are considered the main component of microorganisms in the air. Outdoor fungal spores can be transmitted by visitors, patients, and air (4). High-risk groups may be sensitive or vulnerable to these dangerous biological agents. Fungal bioaerosols should be reduced in hospitals due to high-risk groups. The level of fungal infestation existing in the surroundings is susceptible to an array of factors, encompassing the number of patients and staff in attendance  (5, 6), the influx of external air through accessible gateways or apertures (7), insufficient air circulation, and the refurbishing of clinical edifices (6).

According to research, Aspergillus, Cladosporium and Penicillium species are the most abundant among the different types of fungi in hospital air (8). Aspergillus is the most common cause of fungal infections in hospitals (9), especially in wards that cater to cancer, intensive care, and organ transplant patients. The probability of developing aspergillosis depends on the patient's immune system. Inhalation of even a small amount of some fungal species such as Aspergillus fumigatus can cause death in a person (10), especially for patients in the hospital (11). Certain types of fungi are associated with the sick hospital syndrome, which can cause various allergic diseases. These fungi are typically found in construction dust or dust that has built up in ventilation systems (12-14).

Various factors can impact the hospital environment, including temperature, ventilation, humidity, the presence of organic and nutrient-rich substances, and the materials used in hospital construction (15). Ventilation systems play a significant role in introducing fungal spores into hospital environments. The number of spores present in indoor air varies depending on several factors, including climate, humidity, temperature, ventilation system maintenance, building age, resident movement, and the cleanliness of plants and food sources (16). Molds that grow in the ventilation system or on the interior surfaces of a building are typically species such as Aspergillus and Penicillium (8, 17). Based on a report that was published by the World Health Organization, hospital-acquired infections affect a significant proportion of the  percentage of patients who are admitted to hospitals worldwide, whit rates ranging from 5 to 25 percent across different regions. (18). The prevalence of hospital-acquired infections in developing countries has been reported to be higher than in developed countries. In Iran, hospital-acquired infections are estimated to be between  10 and 15 percent (19). Generally, hospital visitors are the primary source of microorganisms in the air within hospitals. As fungal spores originate from the environment outside the hospital, an increase in the number of residents in a building lead to a corresponding increase in the microbial load within the indoor environment of the building.  Jask port City, located in southern Iran, the combination of high temperatures and humidity creates a favorable environment for airborne microorganisms. This can lead to various healthcare-associated infections among patients and staff due to poor indoor air quality in hospitals. This study aims to examine and analyze the various factors that influence the growth of fungi, including temperature, humidity, the number of patients, the number of patient companions, the number of nurses, and the presence of ventilation and cooling systems in Khatam Al Anbia Hospital, located in Jask City, Iran. The findings of this research can be effective in improving the indoor air quality of hospitals and reducing the concentration of fungal bioaerosols. This, in turn, can help reduce the risk of hospital-acquired infections.

Methods:

Sampling and Determining Sample Size

This descriptive-analytical study was conducted at a Hospital in Jask city, which is affiliated with the Hormozgan University of Medical Sciences. To identify fungal bioaerosols, 43 samples were collected between September and February 2020. Samples were collected from departments of the hospital, including the general and infection emergency, infectious, neonatal, CCU, dialysis, inpatient rooms, as well as internal corridors, and nursing stations. The NIOSH 0800 standard method was used to sample airborne biological particles. Sampling was conducted using an Anderson single-stage air sampling pump, and the cultivation was performed on Subro dextrose agar medium. To facilitate the sampling of fungi, the Biostage 400 was installed on the pump's inlet pipe, and the culture medium plate was placed inside the Biostage. Air was drawn through the culture medium using a suction pump operating at a flow rate of 28.3 liters per minute for a duration of 10 minutes. The bio stage was positioned 150 cm above the ground and placed more than 1 meter away from the wall and window to ensure accurate measurements. Before placing the culture medium inside the sampler, it is important to disinfect and dry the sampling components using 70% ethanol to eliminate any initial contamination.

Identification of fungi:

After sampling, Sabro culture medium plates were incubated under a sterilized hood, and after a week, the colonies grown on the culture medium were isolated in test tubes. All the plates were kept for three weeks because the temperature was colder in this month than the year, and if the fungal colony did not grow, the plates were disposed of hygienically. To accurately identify the samples, the slide culture method, and microscopic observation were used

Results:

The results of fungal proliferation from a total of 43 plates, based on relative frequency, were as follows: Penicillium (36.19%), Cladosporium (29.04%), Aspergillus (21.90%), Scopulariopsis (6.42%), and a specific strain of yeast (8.3%). In the internal department, the most dominant fungi identified were Aspergillus niger and Penicillium. In the emergency department, Cladosporium and Penicillium were the dominant fungi. In the CCU, Penicillium was the dominant fungus. At the time of sampling, the air temperature ranged from 24 to 28 degrees Celsius, and the air humidity ranged from 30 to 58 percent. The humidity data were analyzed in two groups: above 50% and below 50%.

Table 1 displays the concentration density of fungi (CFU/m3) in hospital wards based on a one-way test. As can be seen, the average concentration of fungi in the internal (23.9362 CFU/m3) and neonatal (22.4586 CFU/m3) departments is the highest compared to other departments in the hospital.

The statistical analysis, using the t-test, revealed a significant correlation between the presence of cooling systems (p-value=0.040) and air conditioning systems (p-value=0.002) in the hospital and the concentration of fungi (Table 2).

Table 3 displays the mean and standard deviation of 67 fungal samples, considering variables such as temperature, humidity percentage, number of patients, nurses, and patients' companions.

Table 4 presents the Pearson correlation test between the concentration of fungi in CFU/m3 and various environmental factors. The data in Table 4 demonstrates a significant two-way correlation between temperature and fungal concentration (P = 0.028). In addition, a two-way correlation was observed between the humidity percentage and the number of nurses (P = 0.007). In addition, a statistically significant two-tailed correlation has been observed between the number of hospital patients and the presence of nurses (p = 0.018). Additionally, a significant two-tailed relationship has been established between the concentration of fungi in CFU/m3 and the number of companions accompanying the patients (P = 0.013). A significant two-way correlation was also observed between the number of people accompanying the patients and the number of patients in the hospital (P = 0.028).

Discussion

A total of 67 fungal samples were isolated from 43 plates. The concentration of fungus was found to be high in most areas of the hospital. In the current study, the fungal species Penicillium, Cladosporium, and Aspergillus niger were observed to be the most prevalent. These findings are consistent with a previous investigation conducted by Yousefzadeh et al. (2022) (20) and karimpour (2019) (15). Sautour et al. (2009) also documented the presence of Penicillium and Aspergillus species. These airborne fungi were frequently detected in indoor environments of hospitals (16). Additionally, Kotgire et al. (2020) identified Aspergillus species as well, highlighting its prominence among the fungal species in their studies (21). Similarly, Cladosporium, Penicillium, and Aspergillus were reported as the dominant species isolated from hospital environments. These three fungi have become the most prevalent in hospitals due to their ability to thrive in all types of weather and the small size of their spores, which can remain airborne for extended periods. Based on the data presented in Table 1, it can be observed that the internal ward exhibited the highest prevalence of airborne fungi within the hospital. Conversely, the Infectious Emergency ward displayed the lowest number of fungal colonies. The relatively low pollution levels in the Infectious Emergency ward may be attributed to its heightened significance and sensitivity compared to the other wards. On the other hand, since the sampling time was during the COVID-19 pandemic. Most of the hospital departments, especially the internal departments, were filled with Covid-19 patients and their companions, which is a significant factor in the transmission of fungi. These results are consistent with previous studies that have considered the number and activity of visitors as factors contributing to biological pollution in hospital environments (5, 22). The number of fungal colonies detected in different departments of the hospital was within the standard recommended by the World Health Organization (50 colonies). One possible reason for this could be related to our sampling period, which took place from September to November. During this time, the temperature and humidity of the air tend to decrease. The study conducted by Hollins in 2004 showed a positive correlation between a high concentration of Cladosporium spores and regional temperature (23). El-Sharkawy and Noweir (2014) conducted a study in a university hospital in Saudi Arabia, which revealed a significant correlation between humidity and the density of fungal spores (24).

The t-test analysis revealed a significant relationship between the presence of cooling systems (p-value=0.040) and air conditioning systems (p-value=0.002) in the hospital and the concentration of fungi. Ventilation systems can be classified into two main types: natural ventilation and mechanical ventilation. Mechanical ventilation encompasses both central air conditioning systems and non-central air conditioning systems, such as window units and single-split units. Central air conditioning systems, equipped with filtration systems, are commonly used to eliminate outdoor air pollutants. In the context of hospitals, air conditioning systems can impact the transmission and distribution of infectious agents. In this study the hospital was divided into two sections: the old section and the newly annexed sections. There was a central ventilation and cooling system in the new part of the hospital, but the cooling system in the old parts of the hospital was a split cooler. According to a study conducted by Shajahan et.al (2018), the temperature, humidity, and indoor air ventilation system in hospital buildings have a significant impact on the presence and spread of infectious organisms (25). These factors subsequently influence the outcomes of patients. In a study conducted by Chuaybamroong et al. (2008), it was observed that the concentrations of microbes in units equipped with a single air conditioner were approximately twice as high as those in units with central air conditioning systems. Furthermore, the researchers found that increasing the ventilation rate led to a decrease in microbe concentrations (26). In this study, a significant difference was observed in the concentration of fungi between the old department of the hospital (infectious emergency), which lacked ventilation and central cooling systems, and other departments. And unlike previous studies, it was observed that the concentration of fungi was significantly lower in this section. This could be attributed to the strict adherence to disinfection protocols and the restricted number of patients and visitors in this department.

The results of the Pearson correlation test between fungi concentration (CFU/m3) and environmental factors showed a significant two-tailed correlation between the concentration of fungi in CFU/m3 and the number of companions accompanying the patients (P=0.013) in this study. Our study found a correlation between patients' companions and the presence of fungi in the hospital. The bioaerosol concentration and characteristics may be altered during patient visitation due to the presence of a large number of people, their different activities, and inadequate ventilation. However, a source of fungi in hospitals is not humans, but rather the external environment and other contaminated areas. According to a study conducted by Tang et al. (2009), it was found that the presence of patients visiting hospitals had an effect on the indoor climate and the concentration of bioaerosols in the environment (27). The research carried out by Onmek et al. (2020) in a Thai hospital found that the presence of visitors could be the cause of the elevated levels of bacteria and fungi both during and after their visits. And the number of people in the ward was found to have a positive relationship with the concentrations of airborne bacteria and fungi in Thai hospital (28). Goudarzi et al.'s studies also revealed that there was a rise in the concentration of fungi following visiting hours(29).It is possible that one reason for the spread of fungi in hospitals is that visitors bring them in from the polluted environment outside. This is particularly relevant in our case, as the hospital is located in a tropical port city in southern Iran, where the hot and humid weather, as well as the presence of waste and polluted water, create favorable conditions for microbial growth.

There was a significant two-way correlation between temperature and fungal concentration (P = 0.028) as well.  In contrast to our study, a review study conducted by Hiwa et al. (2021) showed that there was no significant correlation between airborne fungi in hospital environments and temperature (30). The correlation between the concentration of bioaerosol fungi in a hospital and the temperature likely reflects the wide variety of microorganisms present in a hospital setting and their diverse responses to environmental conditions. The research conducted by Molina et al. demonstrated that the presence of fungi indoors in hospitals was influenced by meteorological conditions and external activities surrounding the hospital. Furthermore, the study found a correlation between fungal concentration and both the season and the number of individuals present (31).

Although numerous bacteria and fungi thrive in environments with higher levels of humidity (32), but in this study there wasn’t significant correlation between humidity and concentration of fungi. This finding is similar by Hiwa et al. (2021) study (30). But a research conducted in a Saudi Arabian hospital revealed the correlation between humidity levels and the presence of fungi in various sections of the hospital (24).And also opposite of our study the study conducted by Onmek et al. (2020) showed a significant negative correlation between fungal concentrations and temperature. On the other hand, fungal concentrations were positively correlated with humidity (28).One possible reason may be related to the timing of our sampling, which took place from late summer to winter. During this period, the air humidity generally decreases in Jask, a port and tropical city. The implementation of control measures aimed at preventing and controlling the growth of microorganisms can effectively mitigate the potential health risks posed by airborne microorganisms. In order to decrease the levels of airborne contaminants, it is imperative to adopt internal control measures, such as the utilization of air purifiers and the application of ozone sterilization.

Conclusion

The highest and lowest densities of fungal bioaerosols were found in the internal ward and the infectious emergency ward, respectively. In the internal ward, the concentration of CFU/m3 was 23.9362, while in the infectious emergency ward, it was 7.0922. Following that, the neonatal ward and women's ward had the highest levels of pollutants. The study found that Penicillium, Cladosporium, and Aspergillus spp. were the most common fungal species found in hospital environments. The results demonstrated a significant two-way correlation between temperature and fungal concentration (P=0.028). Additionally, a significant two-tailed correlation was found between the concentration of fungi in CFU/m3 and the number of companions accompanying the patients (P = 0.013). Ensuring appropriate conditions, particularly in terms of temperature and humidity, and adhering to standards for the number of beds per room, are crucial factors in minimizing airborne particles in the hospital environment. Installing an air purifier system, ensuring proper ventilation, and conducting regular maintenance and disinfection of the central ventilation and air conditioning systems are also crucial. Additionally, managing the entry and exit of visitors and companions of patients to hospital wards can further contribute to reducing airborne particles. Further investigation into the variables influencing the concentration of fungi such as all seasons, work shifts, temperature, relative humidity, and the number of patient visitors is suggested, with a particular focus on exploring novel and efficient methods for disinfection and reducing concentration levels.

Limitation

Due to the exorbitant price of the QuickTake pump, an alternative air sampling pump was utilized. Additionally, in order to mitigate the expense associated with the culture medium, the number of samples collected from each medium was reduced from five to three.

Acknowledgement

This article forms a component of a master's thesis in environmental health titled " Investigation of bacterial and fungal bioaerosol concentration in the air of different departments of Khatam Al Anbia Jask Hospital in 2020" Consequently, we extend our sincere gratitude to the personnel of Hormozgan University of Medical Sciences and the staff of Khatam Al Anbia Hospital in Jask City for their valuable cooperation. The authors would also like to express their appreciation to the Persian Gulf Social Determinants in Health Promotion Research Center for their financial assistance.

## Disclosure statement

No potential conflict of interest was reported by the author(s).

Authors’ Contribution

….: Conceptualization, Data Curation, investigation. …Conceptualization, Supervision, Funding Acquisition, project administration. …: Conceptualization, Writing – Review & Editing, investigation. …..: Formal Analysis, Methology. …: Conceptualization, methodology; writing, review, and editing.

## Data availability statement

The data supporting the study’s conclusions are available upon reasonable request.

ETHICS STATEMENT

This study was approved by the ethical committee of HUMS (approval number IR.HUMS.REC.1398.392).

References

1.	Dumała SM, Dudzińska MR. Microbiological indoor air quality in Polish schools. Rocznik Ochrona Środowiska. 2013;15(1):231-44.

2.	Fekadu S, Getachewu B. Microbiological assessment of indoor air of Teaching hospital wards: a case of Jimma University specialized hospital. Ethiopian journal of health sciences. 2015;25(2):117-22. https://dx.doi.org10.4314/ejhs.v25i2.3

3.	Baddley JW, Thompson III GR, Chen SC-A, White PL, Johnson MD, Nguyen MH, et al., editors. Coronavirus disease 2019–associated invasive fungal infection. Open forum infectious diseases; 2021: Oxford University Press US. https://doi.org/10.1007/s12281-022-00434-0

4.	Mosalaei S, Amiri H, Rafiee A, Abbasi A, Baghani AN, Hoseini M. Assessment of fungal bioaerosols and particulate matter characteristics in indoor and outdoor air of veterinary clinics. Journal of Environmental Health Science and Engineering. 2021;19(2):1773-80. https://doi.org/10.1007/s40201-021-00732-8

5.	Aala F, Kohzadi S, Faridi A, Javan K, Amiri M, Ahmadnejad D, et al. Monitoring the air fungal contamination load in two educational hospitals in Sanandaj, Iran. Journal of Advances in Environmental Health Research. 2017;5(4):233-40. DOI: 10.22102/jaehr.2018.84244.1019

6.	Verde SC, Almeida SM, Matos J, Guerreiro D, Meneses M, Faria T, et al. Microbiological assessment of indoor air quality at different hospital sites. Research in microbiology. 2015;166(7):557-63. http://dx.doi.org/10.1016/j.resmic.2015.03.004

7.	Haleem Khan A, Mohan Karuppayil S. Fungal pollution of indoor environments and its management. Saudi J Biol Sci 19 (4): 405–426. 2012. https://doi.org/10.1016/j.sjbs.2012.06.002

8.	Sham NM, Ahmad NI, Pahrol MA, Leong Y-H. Fungus and mycotoxins studies in hospital environment: A scoping review. Building and Environment. 2021;193:107626. https://doi.org/10.1016/j.buildenv.2021.107626

9.	Kauffman CA. Fungal infections. Proceedings of the American Thoracic Society. 2006;3(1):35-40. DOI: 10.1513/pats.200906-040AL

10.	Latgé J-P, Chamilos G. Aspergillus fumigatus and Aspergillosis in 2019. Clinical microbiology reviews. 2019;33(1):10.1128/cmr. 00140-18. doi: 10.1128/CMR.00140-18

11.	Kwon-Chung KJ, Sugui JA. Aspergillus fumigatus—what makes the species a ubiquitous human fungal pathogen? PLoS pathogens. 2013;9(12):e1003743. https://doi.org/10.1371/journal.ppat.1003743

12.	Wahab A, Khamidi MF, Ismail M, editors. An Investigation of mould growth in tropical climate buildings. 2013 IEEE Business Engineering and Industrial Applications Colloquium (BEIAC); 2013: IEEE. DOI: 10.1109/BEIAC.2013.6560139

13.	Luksamijarulkul P, Somjai N, Nankongnap N, Pataitiemthong A, Kongtip P, Woskie S. Indoor air quality at different sites of a governmental hospital, Thailand. Nursing and Palliative Care. 2019;4:1-5. DOI: 10.15761/NPC.1000206

14.	D’Alessandro D, Fara GM. Hospital environments and epidemiology of healthcare-associated infections. Indoor air quality in healthcare facilities. 2017:41-52. doi: 10.1007/978-3-319-49160-8_4

15.	Karimpour Roshan S, Godini H, Nikmanesh B, Bakhshi H, Charsizadeh A. Study on the relationship between the concentration and type of fungal bio-aerosols at indoor and outdoor air in the Children’s Medical Center, Tehran, Iran. Environmental monitoring and assessment. 2019;191:1-13. https://doi.org/10.1007/s10661-018-7183-4

16.	Sautour M, Sixt N, Dalle F, L'Ollivier C, Fourquenet V, Calinon C, et al. Profiles and seasonal distribution of airborne fungi in indoor and outdoor environments at a French hospital. Science of the total environment. 2009;407(12):3766-71. https://doi.org/10.1016/j.scitotenv.2009.02.024

17.	Nasiri N, Gholipour S, Akbari H, Koolivand A, Abtahi H, Didehdar M, et al. Contamination of obsterics and gynecology hospital air by bacterial and fungal aerosols associated with nosocomial infections. Journal of Environmental Health Science and Engineering. 2021;19:663-70. https://doi.org/10.1007/s40201-021-00637-6

18.	Knowlton SD, Boles CL, Perencevich EN, Diekema DJ, Nonnenmann MW. Bioaerosol concentrations generated from toilet flushing in a hospital-based patient care setting. Antimicrobial Resistance & Infection Control. 2018;7(1):1-8. DOI 10.1186/s13756-018-0301-9

19.	Agarwal M, Shiau S, Larson EL. Repeat gram-negative hospital-acquired infections and antibiotic susceptibility: a systematic review. Journal of infection and public health. 2018;11(4):455-62. https://doi.org/10.1016/j.jiph.2017.09.024

20.	Yousefzadeh A, Maleki A, Athar SD, Darvishi E, Ahmadi M, Mohammadi E, et al. Evaluation of bio-aerosols type, density, and modeling of dispersion in inside and outside of different wards of educational hospital. Environmental Science and Pollution Research. 2022:1-15. https://doi.org/10.1007/s11356-021-16733-x

21.	Kotgire S, Akhtar R, Damle A, Siddiqui S, Padekar H, Afreen U. Bioaerosol assessment of indoor air in hospital wards from a tertiary care hospital. Indian J Microbiol Res. 2020;7:28-34. https://doi.org/10.18231

22.	Saadoun I, Al Tayyar IA, Elnasser Z. Concentrations of airborne fungal contaminations in the medical surgery operation theaters (OT) of different hospitals in northern Jordan. Jordan J Biol Sci. 2008;1:181-4.

23.	Hollins P, Kettlewell P, Atkinson M, Stephenson D, Corden J, Millington W, et al. Relationships between airborne fungal spore concentration of Cladosporium and the summer climate at two sites in Britain. International Journal of Biometeorology. 2004;48:137-41.

DOI 10.1007/s00484-003-0188-9

24.	El-Sharkawy MF, Noweir ME. Indoor air quality levels in a University Hospital in the Eastern Province of Saudi Arabia. Journal of family & community medicine. 2014;21(1):39. doi: 10.4103/2230-8229.128778

25.	Shajahan A, Culp CH, Williamson B. Effects of indoor environmental parameters related to building heating, ventilation, and air conditioning systems on patients' medical outcomes: A review of scientific research on hospital buildings. Indoor Air. 2019;29(2):161-76. doi: 10.1111/ina.12531

26.	Chuaybamroong P, Choomseer P, Sribenjalux P. Comparison between hospital single air unit and central air unit for ventilation performances and airborne microbes. Aerosol and air quality research. 2008;8(1):28-36. https://doi.org/10.4209/aaqr.2007.04.0027

27.	Tang C-S, Chung F-F, Lin M-C, Wan G-H. Impact of patient visiting activities on indoor climate in a medical intensive care unit: a 1-year longitudinal study. American journal of infection control. 2009;37(3):183-8. https://doi.org/10.1016/j.ajic.2008.06.011

28.	Onmek N, Kongcharoen J, Singtong A, Penjumrus A, Junnoo S. Environmental factors and ventilation affect concentrations of microorganisms in hospital wards of Southern Thailand. Journal of Environmental and Public Health. 2020;2020. https://doi.org/10.1155/2020/7292198

29.	Goudarzi G, Soleimani Z, Sadeghinejad B, Alighardashi M, Latifi S, Moradi M. Visiting hours impact on indoor to outdoor ratio of fungi concentration at Golestan University Hospital in Ahvaz, Iran. Environ Pollut. 2017;6:62-9. https://doi.org/10.5539/ep.v6n1p62

30.	Hiwar W, King MF, Shuweihdi F, Fletcher LA, Dancer SJ, Noakes CJ. What is the relationship between indoor air quality parameters and airborne microorganisms in hospital environments? A systematic review and meta‐analysis. Indoor Air. 2021;31(5):1308-22. https://doi.org/10.1111/ina.12846

31.	Tormo-Molina R, Gonzalo-Garijo MA, Fernández-Rodríguez S, Silva-Palacios I. Monitoring the occurrence of indoor fungi in a hospital. Revista iberoamericana de micología. 2012;29(4):227-34. https://doi.org/10.1016/j.riam.2012.04.002

32.	Zoz F, Iaconelli C, Lang E, Iddir H, Guyot S, Grandvalet C, et al. Control of relative air humidity as a potential means to improve hygiene on surfaces: a preliminary approach with Listeria monocytogenes. PLoS One. 2016;11(2):e0148418. https://doi.org/10.1371/journal.pone.0148418